MICROSAT TELEMETRY DECODING PROGRAM

TLMDC.EXE version 4 [April 1990]

by Bob McGwier, N4HY

Here is the latest Microsat Telemetry Decoding software.

Note: since April, 1990, the birds have been sending binary telemetry. As a

result, older versions of this program will NOT work.

This program is intended for AMSAT and private use only.  You may re-

distribute far and wide but charge no money, not even handling fees.

Commercial use of this code is absolutely prohibited without permission of

the author.  This code may not be used as the basis of any program that

will be sold, bartered, or in any way traded for gain without the

permission of the author.

There is no warranty. What you see is what you get.

Bug reports to N4HY and I hope you find this usable.

KISSV2.BIN is the KISS eprom image for the TNC-2 and clones, should you

need to burn a KISS eprom.

----------------------------

EQUIPMENT REQUIREMENTS:

You must be running your TNC in the KISS mode.  Full duplex is not required

while in the KISS mode, as this program is for listening ONLY.

A PSK modem for all but DOVE.

Regular BELL-202 modem for DOVE.

A PC, XT, AT or clone machine .

No graphics cards are required.

I make no great claims to the efficiency of the AX.25 algorithm and I hope

that somebody else will jump in and do a better one.

PROGRAM SETUP:

This program is designed for use with COM1 or COM2.  It is written for

simplicity and not elegance.  It does NOT handle framing errors, overruns,

or any such nonsense.  These interrupt handlers are short and sweet and

should be informative to those who have never written one.  I am TSR

friendly.  By that I mean that I hand back.

You must be running your TNC in the KISS mode to capture live data.

The program automatically recognizes which satellite you are listening to

and reads in the appropriate equation file.  This will allow you to update

the files as we refine the equations and not have to replace the program.

Therefore you must have the files

PACSAT

DOVE

WEBER

LUSAT

in the current directory on your disk at ALL times, especially if you are

going to `tune around' amongst the different satellites the first few days.

If you have brought the program up, you will see data being dumped to the

screen when the bird is tuned in.

The program comes dialed in for COM1:9600,N,8,1.  You may change the speed

and the COM port on the command line.  To set the COM port and speed at

which the serial port is run, when you bring the program up type

TLMDC COMn:4800,N,8,1   (where n is the serial port: 1 or 2)

This will set it up at 4800 bps.  At 2400 BPS you are really pretty slow

and at 1200, I have my doubts whether or not you will stay ahead of WEBER's

long telemetry list if you are using a plain XT.

PROGRAM OPERATION:

You may want to invoke TLMDC4  with a batch file to automate the entire

progess. Here's an example of one called DECODE.BAT:

C:\> DECODE.BAT

TLMDC COM2:4800,N,8,1

^Z

this invokes TLMDC to run at 4800 baud on COM2:

Once you've brought up the program, there are four main functions you can

select by touching a key.

They are

D = Toggle Dump, T = Tlm Decode, F = Decode File, Q = Quit

as seen from the main page.

D dumps the incoming frames (all of them) to a file.  Both a raw data file

and an ASCII file are required.  You will be prompted for them.  While

you are seeing the frames on the main page, or decoding telemetry, the

data will be dumped.

The ASCII data file is primarily for you to be able to eyeball the stuff

that came in.  The raw file is of course full of unprintable characters

(shifted address characters, control info, etc.). T starts decoding the

telemetry.

F will decode the frames from a previously captured RAW data file.

Q leaves the program.

When you type  T   the screen will be cleared and the rudimentary telemetry

decoding, based on the preliminary equations will occur.

When you type  D   for the first time the screen will be cleared. You will

be asked for two file names, one for a raw data file and one for ASCII

dump.  [ASCII dump is what I included below].  RAW is what you will need

for function F.

When you type  F   the screen will clear and prompt you for the name of a

RAW telemetry dump file.  This program will not read ASCII files like the

ones below. It will dump complete frames at a time, uptime, both telemetry

frames, and a status line and the stand by for you to touch a key.  When

the end of file is reached, the message in the bottom right hand corner

changes and says end of file touch a key and you will be back at the main

menu.

Touch any key and the program will return to the main page (possibly after

a minor delay in timing out the ax.25 receiver).

Below is an example of the old ASCII format telemetry.

LUSAT -1>TIME  -1 V   UI: PID=F0

PHT: uptime is 000/13:38:49.  Time is Mon Jan 15 21:21:30 1990

LUSAT -1>TLM   -0 V   UI: PID=F0

00:75 01:75 02:7A 03:75 04:7A 05:7D 06:75 07:80 08:7D 09:75 0A:7A

0B:7D 0C:75 0D:7A 0E:7C 0F:75 10:75 11:75 12:75 13:77 14:75 15:75

16:7C 17:77 18:75 19:77 1A:7D 1B:75 1C:75 1D:75 1E:75 1F:76 20:75

LUSAT -1>STATUS-0 V   UI: PID=F0

00 00 00 80 B0 18 55 03 00 00 00 00 30 00 00 00 00 00 00 00

LUSAT -1>WASH  -0 V   UI: PID=F0

wash addr:2800:0000, edac=0xc0

LUSAT -1>TIME  -1 V   UI: PID=F0

PHT: uptime is 000/13:39:00.  Time is Mon Jan 15 21:21:41 1990

UPTIME is the number of days/hh:mm:ss since the kernal was started.

WASH is the output of the memory wash routines, which will correct errors

in the stored memory.

STATUS lines output various status bits.  The first three are receiver

digital port bits, the next three are Power module digital port set bits,

and the next three are the transmitter digital port set bits. The next

three are used on DOVE and PACSAT to give the digital port bits for the

DOVE and S band modules respectively.  In the thirteenth position, the

seperation switch is sensed.  If you see this, the high order bit (0x80)

will always be or'd with the byte since the transmitter won't be on

otherwise!

UI means it is a UI (unnumbered information) frame.

PID=F0 is the protocol ID byte and F0 is RTTY-PACKET (a derogatory term for

regular TNC packet).

C0 is TCP-IP, etc.

V followed by a blank means that there was no digipeater field.

[Of course there won't be with the bird BUT, the algorithm may be used to

monitor regular packet and will dump the other type frames as well.  There

just won't be a satellite id'd or telemetry decoded.]

The status decode will vary from one bird to the other, with some

consistency.

The TXA means transmitter A and TXB means transmitter B.

0 means off and 1 means ON.

SBT is the S band transmitter.

BCR is the value set on the digital to analog converter (DAC) in the power

module.

DAC controls the battery charging rate.