SatSked version 1.966               8/17/1996

WA2N and NZ3F

SatSked's Function:

-------------------

SatSked is a multiple satellite pass manager for amateur radio enthusiasts.

It is used to automatically track different satellites by switching various

ground station equipment via computer.  It is intended to be used with the

programs PB, PG, and their associated files.  All of these programs may be

obtained from AMSAT USA.

SatSked requires a registration number to run.  This number is available

from AMSAT NA, UK and DL.  Each of those organizations has set their own

price for SatSked.  All funds from SatSked go directly to the Phase 3D

satellite construction fund.  You may reach AMSAT at the following addresses:

Radio Amateur Satellite Corp  (AMSAT NA)

850 Sligo Ave

Suite 600

Silver Springs Maryland

USA

phone 301-589-6062

fax   301-608-3410

internet:  martha@amsat.org

AMSAT-DL Warenvertrieb

Reinhard Richter

Lohfeldweg 40

D-30459 Hannover

AX.25 DJ1KM @ DB0FAU.DEU.EU

CI$ 100114,756

Fax +49 511 411112

Ron Broadbent

G3AAJ

AMSAT UK

E12 5EQ

England

phone 44(0)81 989 6741

fax   44(0)81 989 3430

SatSked Calculates The Future Times of AOS / LOS:

-------------------------------------------------

SatSked creates it's own scheduling information.  The user's latitude,

longitude, and station elevation are used to determine the AOS and LOS times

of the satellites listed in the file SATSKED.PRI.

A file containing keplerian elements of the satellites to be tracked is

necessary to make these calculations.  This file may be in NASA 2 line, or

AMSAT format.  The name of the keplerian file is set in the SATSKED.CFG

configuration file.  This file name can use wildcards, so that the latest

version of the elements will automatically be chosen.  This is described in

the text of SATSKED.CFG.

Where To Put The Distribution Files:

-----------------------------------

First designate a single directory where you will keep the group of files for

SatSked.  This directory should be added to the dos search path.  You should

copy all of the files from SATSKED.ZIP into to that directory.  In my

examples, I'll use C:\SATSKED as the "SatSked" directory.  Refer to

Appendix C: at the end of this document for a suggested set of files, and

placement, to track satellites with SatSked, and PB/PG or SatLink by NZ3F.

How SatSked Works:

-----------------

SATSKED.EXE watches the dos clock until the proper time and writes a file

named TRACK.BAT to disk.  The contents of TRACK.BAT is dependant upon the

satellite to be tracked.

SatSked then saves it's entire program memory to disk, extended memory, or

expanded memory.  The now unused memory allocation is then given back to

DOS to make room for other programs that are about to run.  Next SatSked

shells to DOS, and runs TRACK.BAT as a "child process".  After TRACK.BAT

finishes it's commands, SatSked retrieves it's memory allocation from DOS, and

restores the image that was saved to disk, extended memory, or expanded memory.

The choice of where the image is stored is selectable with the swapType

parameter, in SATSKED.CFG.

When TRACK.BAT is executed, it calls the appropriate batch files

for the satellite to be tracked.  From TRACK.BAT, batch files that start your

rotor radio tuning, and STRTPBPG.EXE can be called.  The program STRTPBPG.EXE

(included with SatSked) will then will start either PB.EXE or PG.EXE,

depending on the presence of files to be uploaded to the satellite.  You may

also run other programs than STRTPBPG, PB and PG, depending on your

configuration for a particular satellite.

SatSked will now operate directly with SatLink by NZ3F to communicate with

the satellites.  SatLink is replacement for both PB and PG in one program

which allows simultaneous FTL0 uploads and broadcast mode downloads.  SatLink

also negates the need for STUFFIT.COM to assure proper termination of SatLink

at the end of the satellites pass.

Batch File Explanation:

----------------------

Batch files are command files which have the extension .BAT.  They are

created with any text editor.  Batch files can be used to execute DOS

commands, and perform a limited set of other operations.  A batch file can

be accessed from anywhere on disk, as long as the directory it is "lives in" is

part of the dos search path, set in AUTOEXEC.BAT.  I have a c:\batch

directory that contains several batch files with short names.  This allows me

to move to often used directories quickly, and  also to execute programs that

are not in the dos search path.

TRACK.BAT:

---------

This is the file that SatSked generates.  Track.bat will be located in the

C:\SATSKED directory (or where you have SatSked located) when SatSked exits.

TRACK.BAT is also used to start the idle task function if it is enabled.

(see Idle Task: below)

*****TRACK.BAT**********************Comments:*********************************

CALL UO-22[C].BAT [X local_end_time unix_style_time inject_i_time maxPgCycles

abortIdleSeconds next_aosTime strtpbpgTimeDelay

renameXULtoOUT]

Calls the proper batch file to start tracking

the specific satellite UO-22.  If KO-23 was

the satellite to track, KO-23.BAT would

replace UO-22.BAT here.  In addition, it should

change the default directory to that which

contains the proper PB.CFG and PG.CFG.  If

SatSked is started with an ASCII parameter,

such as "SATSKED.EXE W", the bracketed C will

be replaced with W.  This allows a different

set of batch files to be utilized with SatSked,

giving you a way to have two or more setups,

say one for dos, and one for windows.  If no

parameter is present on the command line,

UO-22.BAT would be called.

The other bracketed parameters only are sent to

the batch file if the STRTPBPG flag is set to 0

for the appropriate satellite in SATSKED.PRI. In

this case, SatSked expects you to start your own

ground communications software in your batch

file.  The parameters are present for your use

in ending the pass and returning to SatSked.

The parameter X will be A for an abort time, L

for a normal LOS time, I for abort when Idle,

or N (for none) if you have manually started a

pass with the <S> key in SatSked.  The

local_end_time is the Gregorian formatted

local time that SatSked has determined that

the pass should end.  unix_style_time is that

same ending time in units of seconds since

January 1, 1970.  inject_i_time is the number

of seconds after a pass has started to inject

an "I" keystroke, which will satisfy the

EXITAFTER feature of PB.EXE.  The maxPgCycles

parameter is a copy of that entry in

SATSKED.CFG, and is intended to be used to

count how many times PG.EXE should cycle while

trying to upload a file to the satellite.  The

abortWhenIdle parameter is the number of

seconds after PB has obtained an up to date

directory, and is idle, to abort and go back

to SatSked.  This will only happen if an equal

or lesser priority satellite's pass is

conflicting. The next_aosTime parameter is used

to prevent SatSked from abortingWhenIdle until

the next satellite to be tracked is above the

horizon.  strtpbpgTimeDelay is the time in

milliseconds that STRTPBPG waits while

displaying status messages.  renameXULtoOUT

will be set to 1 if it is desired to rename

any .XUL files to .OUT files that are created

by a satellite rejection of an upload.

[STRTPBPG X local_end_time unix_style_time inject_i_time maxPgCycles

abortIdleSeconds next_aosTime strtpbpgTimeDelay renameXULtoOut]

This line is present ONLY if the Comm Pgm

column of SATSKED.PRI is set to 1 for the

appropriate satellite.  STRTPBPG then starts

the TSR STUFFIT.COM, which prepares a timed

keystroke.  This keystroke is what will

prevent PB or PG from "hanging" if no satellite

down link packets are ever heard.

Next STRTPBPG.EXE runs PG, if there exists an

.OUT,.PUL or .XUL file in your satellite

directory, and the file PG*.CFG exists.

This is described in detail                                 below.

Note that the parameters X through

renameXULtoOUT have the same meaning as

described above.

if errorlevel 1 goto no_pb

This line is present ONLY if the Comm Pgm

column of SATSKED.PRI is set to 1 for the

appropriate satellite.

To conserve cpu memory when running PB, a

batch file will be created by STRTPBPG.EXE,

which calls PB.  If the S key is hit while

STRTPBPG is running, or the LOS time or abort

time is exceeded STRTPBPG will exit with an

error level of 1.  In this case, SPB.BAT is

skipped, and PB is not run.

call spb.bat

This line is present ONLY if the Comm Pgm

column of SATSKED.PRI is set to 1 for the

appropriate satellite.

As mentioned above, to conserve memory SPB.BAT

is called to start the PB program.  This saves

several tens of kilobytes which are required

by STRTPBPG.

:no_pb

This line is present ONLY if the Comm Pgm

column of SATSKED.PRI is set to 1 for the

appropriate satellite.

Executions branches here if PB.EXE is not

to run.

[SATLINK configFile pass_seconds abortIdleSeconds optionalParameters]

This line is present ONLY if the Comm Pgm

column of SATSKED.PRI is set to 2.  The

program SatLink by NZ3F will be used as the

comm program for the appropriate satellite.

The first parameter, configFile, is the

path to SatLink's configuration file for

the satellite about to be run.  This string

is extracted from the file SATLINK.DIR, if it

exists, AND there is an entry which matches the

name of the satellite about to be run.  If

SATLINK.DIR is not found in the SATSKED

directory, or a match to the bird's name

cannot be made, the file name takes the form

SLxxxxx[c].CFG. The xxxxx's will be replaced by

the satellite name, and the C parameter will

be present if SatSked is started with an ASCII

parameter, such as "SATSKED.EXE S", then the

bracketed C will be replaced with S.

The following parameters passSeconds, and

abortIdleSeconds and are no longer used

by versions of SatLink distrubuted on or after

4/19/95, but are included for compatibility

with older versions.  Similar information is

passed to SatLink through the disk file

exitinfo.dat.

Pass seconds will be non zero if an abort time

is present for that pass, and will indicate

the number of seconds that SatLink should

run before terminating.  The abortIdleSeconds

parameter is the number of seconds after SatLink

has obtained an up to date directory, and is

idle, to abort and go back to SatSked.  This

will only happen if an equal or lesser

priority satellite's pass is conflicting.

The last parameter "optionalParamters" is

created by the third column in the file

SATLINK.DIR.  Read SATLINK.DIR for more

information.

call ssclean[C].bat             This batch file can be used to terminate

the antenna / tuner tracker, clean out

old .DLX messages, call postpass.bat or

whatever you want to do after each pass.

A master ssclean.bat can be located anywhere

in the searchpath.  You may also override this

master with a local copy in any of the

satellite directories.  This allows you to do

a different type of cleanup for each satellite.

The bracketed C character is replaced with

SatSked's command line parameter if present,

and is omitted if not.

*****************************************************************************

Note that SPB.BAT is created automatically by STRTPBPG.

*****SPB.BAT************************Comments:*********************************

pb.exe                          This line starts PB.EXE.

stuffit /r                      This line removes stuffit from memory

******************************************************************************

SATELLITE_NAME[C].BAT:  (The bracketed character C is only present, and is set

---------------------    to the first character of the first parameter on

SatSked's command line)

These files should be located in the SatSked directory.

***UO-22.BAT**********************Comments:***********************************

c:

cd \pb\uo22                     This changes the default drive and directory to

the one  where the specific PB.CFG and PG.CFG

exist for UO-22.  You must use the directory

names and drive for your particular setup here.

call RUNUO22.BAT                A batch file that starts your rotor tracking,

radio tuner, and whatever else you need to do

before  PB/PG is run.

******************************************************************************

RUNUO22.BAT, is specific to my system, so I won't include a copy of it here.

Basically I use a program to change my radio's to the proper mode, USB

RX, FM TX for the psk birds, and FM on both for the 9600 bps satellites.  I

set my doppler correction program's base frequency to that of the satellite

uplink and downlink in order to compensate for doppler shift, and finally

change to the proper modem mode using the DSPMODE program for the DSP-12.

Your requirements may vary from mine.

If STRTPBPG is set to 0 in SATSKED.PRI, then 4 parameters are sent to the

SATELLITE_NAME[C].BAT file.  This file will be responsible for running the

ground station software.  See the section on TRACK.BAT for details.

STRTPBPG.EXE:

------------

SRTPBPG.EXE should be located in a directory that is in the dos searchpath.

Hopefully you have put the SatSked directory in the searchpath, and this is

where you will put STRTPBPG!  This will enable execution from any satellite

directory without specifying the exact path.

When TRACK.BAT is called from SS.BAT after SatSked exits, it starts up your

rotors, radio tuner, and changes to the proper satellite directory through

the call to UO-22.BAT, or another satellite specific batch file.  Next,

LOS or Abort time information is passed to STRTPBPG.EXE.

STRTPBPG searches the satellite directory for .OUT and .XUL files.  If it

finds one, it runs PG.EXE -U -Q. If not, it starts PB.EXE as described below.

To avoid getting stuck in PG.EXE if the satellite never comes, or a slot

doesn't open, STUFFIT.COM is called before PG.EXE.  Depending on the commands

from SatSked, STUFFIT will "press" the Esc key at the ABORT or LOS time.  This

will return control from PG.EXE to STRTPBPG.EXE.

If a .XUL file is found from a reject of an upload, and renXULtoOUT is set in

SATSKED.CFG, the .XUL will be renamed to .OUT.  If more files exist to be

uploaded, STUFFIT.COM will be reinitialized and PG.EXE will run again.  This

process will repeat "maxPgCycles" times, or until the LOS or ABORT time is

reached.  This feature must be used carefully, and maxPgCycles should not be

set too high. maxPgCycles is set in SATSKED.CFG, and defaults to the value 5.

If there is time left before LOS, SPB.BAT is created, then is called to start

PB.EXE.  Also, STUFFIT.COM is called to press the "I" key or "Q" key from

within PB.  If no abort time is present, "I" will be pressed after the

parameter injectI seconds after PB.EXE is run.  This will satisfy the

requirement that packets are heard by PB.EXE, so PB's exitafter command will

function.  If an ABORT time was present when STRTPBPG was started, the "Q" key

will be pressed at the proper time so SatSked will regain control.

The default action of STRTPBPG may be modified by pressing the S key to

return to SatSked, or pressing the B key to jump directly to PB.  Note if

you have set a very short time for strtpbpgTimeDelay, it will be very

difficult to press either of these keys in time to change operation.  You

can however press the keys just before STRTPBPG runs, and they will be

heard by STRTPBPG.

STUFFIT.COM:

-----------

STUFFIT.COM should be located in a directory that is in the dos searchpath.

This will enable execution from any satellite directory without specifying the

exact path.  Once again, a good place would be in the SatSked directory, if

you have placed it in the dos searchpath.

The program  STUFFIT.COM, is a "TSR", or "terminate and  stay resident" program

that "presses keystrokes at a predetermined time.  If a keystroke is not hit,

and no packets are heard, PB will never exit regardless of the EXITAFTER value

that is set in PB.CFG.  I use the value "1" for EXITAFTER in my PB.CFG file.

PG.EXE has no exitafter command, this is why the Esc key is pressed at the

ABORT or LOS time.  If you run stuffit with a /? parameter, a short usage

statement appears.  One feature I've found useful is the ability to execute

commands from a disk file with the /f:filename switch.  This way several

keystroke events can be stacked up in time.  This could be useful when playing

around with FO-20 or another "non-standard" satellite.  For more information,

see STUFFIT.DOC.

Note that STUFFIT counts time through clock ticks.  Unfortunately, if you are

running DesqView, or Microsoft Windows, clock ticks are "stolen" to swap from

task to task.  This means that STUFFIT won't get 18.2 clock ticks per second,

and it's internal time will run slow.  This results in the injected keystroke

coming later than desired.

TRACK.BAT Examples created by SatSked:

----------------------------------------

Here is an example of a track.bat that was generated for UO-22.  Here I have

started SatSked with a parameter "w" (SATSKED.EXE W).  The STRTPBPG column

in SATSKED.PRI was set to 1, and this is a normal pass with an LOS local time

of 14:30:24:

call  uo-22w.bat

strtpbpg L 14:30:24 737058594 80 10 5 737061111 2500

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call sscleanw.bat

Here is track.bat which was started manually with the S key.  No command line

parameter was entered when I ran SatSked.  Also the STRTPBPG parameter was

set to 0, so lo-19.bat is responsible to start the ground communications

software, start stuffit etc.

call lo-19.bat N 00:00:00 737109278 80 10 5 73719501 2500

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call ssclean.bat

Here is track.bat with an Abort time of 14:48:00 local.  The abort function

is flagged with the "A" parameter.

call  ao-16.bat

strtpbpg A 14:48:00 737059650 80 10 5 737059812 2500

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call ssclean.bat

Here is track.bat which was generated when a conflicting pass of equal or

lesser priority exists.  The "I" parameter tells STRTPBPG to start stuffit so

that it will press the "Q" key when the up to date, idle state is reached in PB.

Note that SatSked will cycle back to the same satellite if the AOS time of the

conflicting bird has not yet been reached.

call io-26.bat

strtpbpg I 23:28:16 753856069 80 10 5 753856350 2500

if errorlevel 1 goto no_pb

call spb.bat

:no_pb

call ssclean.bat

This is a track.bat file which was created to run UO-22 using SatLink as the

comm program.  An abort time was present, and UO-22 was to be run for 6 minutes

and 9 seconds before aborting.

call uo-22s.bat

satlink sluo-22s.cfg 369 0

call sscleans.bat

ABORT Times:

-----------

If SatSked has determined that there is a conflict with the subsequent

satellite pass which has a higher priority, STUFFIT.COM is called (through

STRTPBPG.EXE) with the parameters  =HH:MM:SS 'Q', where HH:MM:SS is the AOS

time of the next satellite.  This quits PB, saves all data downloaded to disk.

When we get back to SatSked, the program notices that the current time is

beyond the "abort time" and it moves on to the next pass.  Now the process

repeats with the next satellite.  The level of searches done to check priority

is equal to the number of "priorities" loaded from the file SATSKED.PRI to be

described later.  To abort, the next satellite must have a HIGHER priority than

the first.

If the parameter abortEarly is set to a non zero value, then the abort time

written to pass.txt will be modified by the number of seconds specified.  To

abort earlier than the AOS time of the higher priority satellite, use a

positive number.

Idle Tasks:

----------

In between satellite passes, it is possible to run an external program, and

use stuffit.com to exit that program in time to run the next satellite pass

from SatSked.  This is done by selecting a non zero value for the SATSKED.CFG

parameter idleTaskRunSeconds.  This parameter is the minimum number of seconds

from the current time, to the start of the next pass, which will allow the

idle task to be started.  If the length of time is shorter than that specified,

the idle task sequence will not be initiated.

SatSked will then create TRACK.BAT, which contains a call the batch file

IDLETSK.BAT, and then exit.  To facilitate returning to SatSked in time to

start the next satellite pass, several parameters are fed to IDLETSK.BAT.

The format is:

CALL IDLETSK[C].BAT +T1 T2 T3 SATELLITE_NAME

where

C is the first parameter when SatSked is started, such as "SATSKED.EXE W",

the bracketed C will be replaced with W.  If no parameter is entered,

the IDLETSK.BAT will appear.  Note that the brackets do not appear.

+T1 is the offset time in hours:minutes:seconds until the next pass.  This

string is properly formated to be used as a direct parameter to

STUFFIT.COM to delay execution for that length of time.

T2 is the absolute local time in hours:minutes:seconds until the next pass.

T3 is the number of seconds until the next pass.

SATELLITE_NAME is the ASCII root name of the batch file which will be

called when SatSked is restarted at the proper time.

Example (KO-23 is due in 11 minutes and 52 seconds at 00:54:06 local dos time),

here is the contents of TRACK.BAT:

call idletsk.bat +00:11:52 00:54:06 712 ko-23

Note that the times are earlier than the actual satellite's AOS by beforeAOS

seconds, as set in SATSKED.CFG.

Here is an example of IDLETSK.BAT that you might use to run TCPIP between

satellite passes, and use stuffit to type "exit" to leave the program:

stuffit %1 'exit' 13

net.exe

stuffit /r

Note: W5RKN advises that this sequence will remove you from "NOS" net:

stuffit +20 13 +5 13 +5 13 %1 @68 +5 'exit' 13

net

stuffit /r

Another possibility is if you want to use your own screen saver, such as

RAIN.EXE, which uses the escape key (27) to exit:

stuffit %1 27

rain.exe

stuffit /r

If you wish to start the Idle Task manually, use the "j" key.

SatSked's Configuration, And How To Change It:

---------------------------------------------

The file SATSKED.CFG contains a detailed description of configurable

parameters, and their default values.  This can be edited before you start

SatSked, or from within SatSked by pressing the "C" key.  This will drop

you into the default text editor TED3.COM, and load SATSKED.CFG.  You can

change any parameter by typing it's name followed by your choice.  When you exit

the editor, SatSked will scan this file and update it's parameters to those in

SATSKED.CFG.  If you comment out a parameter with a ";" at the beginning of

the line, it's value will not change back to the default until SatSked is

exited, and re-initialized.

Setting Priorities For Each Satellite in SATSKED.PRI:

----------------------------------------------------

SatSked goes through a "bumping" procedure when 2 or more satellites are

above the horizon at the same time.  The satellite that gets bumped is the

one with the lower priority.  These priorities are set by editing the file

SATSKED.PRI.  This too can be reached from within SatSked by pressing

the "P" key.  When you do this, and exit the editor, SatSked will recalculate

all "abort times" in the program.  If you had previously manually changed

the "abort times" in the PASS.TXT file (described next), these changes

will be lost.  Higher numbers indicate a higher priority.  The bumped pass is

still displayed, and will have an "abort time" entry in the pass window.

Parameter fields in SATSKED.PRI:

--------------------------------

The first column is the satellite's ASCII name.  This should match the name

in the keplerian elements if automatic AOS/LOS time generation is desired.

Character case is not important.  If there are spaces in the name in the

keplerian elements file, use quotes as shown below.  Note that the spaces

will be replaced with the tilde character (~) when displayed in the AOS/LOS

window, and when track.bat is generated for this satellite.  The satellite

priority is the next column, and valid values range from 0 to 9.  If there

is a 0 in this column, a pass will be not be started.  However, the beeper

will run to let you know that the satellite is present.

The next parameter is the maximum number of runs for that satellite.  The

parameter maxRuns is the value "maximumRuns" in SATSKED.CFG. This tells SatSked

how many times to rerun this particular pass if control is returned to SatSked

before the satellite falls below the horizon.

STRTPBPG is set to 1 if you desire to use the STRTPBPG.EXE program that I have

supplied to start your PB and PG programs, or 2 to utilize SatLink by NZ3F.  If

you want to specify your own comm program in your satellite batch file, then

set this parameter to zero.

The next parameter is new to SatSked versions 1.948 and later.  This is

minimum elevation for only that satellite.  This elevation in degrees will be

used as your local "horizon" value when generating pass data.  Also affected

will be indications that the bird is above the horizon in the satellite window.

Next is the Hours and minutes in cpu local time to allow passes to run. Any passes

that occur outside of this time will be skipped.  If blank, all passes will be

suppressed.

Next is the path to the upload directory for messages to users on this

satellite.  If the upload path is not present, you will not be allowed to send

a message.

If you want to manually start an event, like starting your antennas tracking a

voice satellite, you may put the root name of a batch file in the name column

instead of a satellite name, like the RAO13 entry in the following example.

This would then be actuated by pressing the S key to start a pass, and

selecting the entry corresponding to RAO13.  To avoid confusion when sending

a message, leave the message path column blank for these entries.

An example of SATSKED.PRI is shown below:

;name           pri  maxRuns  Comm Pgm minElev   run time   message path

;----           ---  -------  -------- --------  ---------  -------------

AO-16           2       3       2       3        0000-2359  \satsked\ao16

lo-19           1       3       2       3        0000-2359  \satsked\lo19

UO-22           6       3       2       0        0000-2359  \satsked\uo22

KO-23           4       3       2       0        0000-2359  \satsked\ko23

KO-25           5       3       2       0        0000-2359  \satsked\ko25

io-26           3       3       2       0        0000-2359  \satsked\io26

"Courier 1B r"  0       1       0       0        0000-2359

"noaa 10"       0       1       0       0        0000-2359

Automatic AOS / LOS table generation:

------------------------------------

Thanks to Jesse Buckwalter NZ3F, SatSked generates it's own AOS / LOS

times.  When using a math co-processor, this happens very quickly at

program start up.  If you don't have a co-processor, it may take several

minutes to generate new passes.  If the table (pass.txt) contains at least

one pass, then pass table generation will stop in time to execute the next

pass.  If you're starting SatSked for the first time, then make your

kepSearchHours setting small to make sure you quickly get a calculation

for each satellite.  If the program exits because no passes are calculated,

increase the setting of kepSearchHours long enough for a bird to be found

above the horizon. An AOS then, LOS time must be found in order to qualify for

storage.  Therefore, Satellites passes that are above the horizon when the

calculation starts are not added to the pass table.

SatSked allows you to change the elevation at which it considers to be the AOS

or LOS crossing.  An array of 8 parameters, which is specified in SATSKED.CFG

is called genKepMinElevation.  The 8 parameters represent the local horizon,

split up into 8 segments of 45 degrees each.  These values are entered

in positive degrees to delay AOS and negative accelerate LOS.  The first number

which follows genKepMinElevation covers headings 0-44 degrees, the second

45-89 degrees etc.  If you dont enter 8 values, the last value entered will

be used for the remaining headings.

When SatSked starts, it looks at the number of passes loaded.  If this number

is less than the parameter "sortGenThreshold", SatSked will start adding to

the pass table automatically for each bird in SATSKED.PRI which has a non

zero priority, or all of them if "genKepZeroPriority" is set to 1.  In this

case, the name must match one of the names in the keplerian element file.

While the pass generator is chugging away, it is watching for the time of

the next pass.  If this time is exceeded, it gives up and discards that

partial pass, beeps, creates track.bat and exits as usual.  Thus, even though

it's slow, you should never miss a pass due to slow calculations.  If you

happen to be at the CRT during this, and want to quit, just hit a key.  The

last set of passes completed will be added to your pass table (pass.txt).  Note

that some of the birds may not have been gotten to, this is ok.  The next time

the number of passes drops below "sortGenThreshold", and you leave the

keyboard alone, SatSked will pick up where it left off for all birds, and

compute to "kepSearchHours" past the last valid bird in the table (any bird)

for all satellites.

You may want to play with these two parameters to make this convenient for

your particular operation.  Note that entries you have manually edited in the

pass table won't be affected, unless n passes near the end of the pass.txt

file.  N is the number of non zero priority satellites in SATSKED.PRI, or

the total number of birds in that file if "genKepZeroPriority" is set.

Additional passes may be generated by pressing <G> then selecting <1> for

"append to present".  If you wish to replace all the passes, select option <2>.

Printing Out a Satellite's Azimuth and Elevation Values:

--------------------------------------------------------

From the pass generation menu, selecting item 3 will allow you to investigate

the azimuth and elevation values of any pass in the pass window.  Just enter

the desired pass number (left most number in the pass window), the step time

in seconds (defaults to 60 if carriage return is hit with no value).  If you

wish to print just to the screen, hit carriage return again, or to print,

hit 1 and then carriage return.  An azimuth and elevation value is printed

to the screen (and printer) for each time step.  The screen will pause after

every 18 values, hit the carriage return to see the next 18 values.

Manually Changing AOS, LOS and Abort Times in the PASS.TXT file:

---------------------------------------------------------------

When the priorities have been scanned, the "Abort" times are stored in the

last column of the PASS.TXT file.  If you wish to change these times, or

any of the AOS or LOS times, or delete a pass entirely, you may edit

PASS.TXT.  Once again, this can be done from within SatSked by pressing

the "E" key.

Be careful what you edit!  SatSked expects it's times at the proper column.

When you exit from the editor, SatSked will immediately use the new times you

entered.

Keplerian Window:

----------------

Up to 100 satellites may be tracked by SatSked.  Each of the satellites in

the keplerian data file may be displayed.  In the normal display mode,

the satellites are displayed as they are found in the file KEPORDER.TXT.  This

file should contain just the name of the satellite to be displayed, one bird

per line.  If the bird's name contains a space, put double quote characters

around the entire name.  Example:  "DEEPSPACE 9"

Following the name of the bird is an optional parameter which will cause the

AOSLOS display field to be blanked out for an entry of 1, or replaced by the

satellites altitude in meters if the number 2 is entered.  This can be useful

to avoid the long computation times necessary to find the horizon crossing

time of individual satellites.  Edit that file with a text editor to change

the display order.

The size of the real time tracking window is determined by what's left over

after the pass window size is specified with passesToDisplay.  The delta E

indicates the direction of change of the elevation by an arrow up or down,

and the dR/dT is the rate of change of the range from you to the satellite

in Kilometers / Second.  This is used for doppler calculations.  Use the

control key with the up arrows, page up, page down, control and home keys to

scroll through the satellites.

If displayAOSLOS is set to 1, the rightmost column will display the hours,

minutes, and seconds to LOS or AOS of that satellite.  Performing this

floating point calculation is VERY cpu intensive, and I don't recommend it

unless you have a coprocessor.  However, the calculations only are made once

for each horizon crossing.  The values are saved on disk when SatSked exits,

and restored when the program starts back up.  When paging through the

keplerian window with control arrow/home/end/page keys, there will be a

delay when any satellite that was not present in that window suddenly appears.

If horizonBeep is set to 1, then SatSked will send a rising or falling tone

to the CPU speaker.  This will occur even with the screenSaver, but the time

of occurance may be off by as much as 30 seconds or so.  This is due to the

fact that the position of one satellite per second is computed while the

screen saver is operating.

Note that the horizon crossing elevation can be non zero, satellite

specific, and depend on the azimuth.  The values default to zero, but

can be changed in SATSKED.PRI per satellite, and with 8 genKepMinElevation

values in SATSKED.CFG.  The 8 genKepMinElevation values break the horizon

into 8 equal sized segments (45 degrees).  The edges of these segments

are smoothed, to prevent problems when a satellite crosses from one segment

to another.

OrbitDRV Window:

---------------

This window will appear if SatSked detects the presence of OrbitDRV or RotorDRV.

Both of these programs are TSR's that are used to control rotor movement.  There

are two lines of information displayed, here is a list of the display items:

1.  Ta and Ca are target and current azimuth,

2.  Te and Ce are target and current elevation.

3.  A indicates bad station elements.

4.  E indicates bad satellite elements.

5.  F indicates the rotor is in flipped mode (if using KB5MU's FLIPANTS pgm).

6.  R indicates that RotorDRV has been enabled,

7.  T indicates that OrbitDRV's tracking is enabled.

8.  U indicates that OrbitDRV's tuning is enabled.

9.  D indicates that RadioDRV is being driven directly every second from SatSked

without the intervention of OrbitDRV.

10. I indicates that the KCT software has detected an I/O error with the KCT

board.

11. S next to Ca or Ce indicate that the rotor has stalled.

12. The arrows indicate the direction of rotor movement.  Note that they ae

sampled once a second, and small movements may not be indicated.

13. The name of the satellite tracked will appear as no data if either the

station elements are bad, or the satellite elements are bad, and

SatSked is not driving RotorDRV direct.  The bird name is shown

otherwise.

Site Longitude Entry:

--------------------

Jesse NZ3F wrote the orbital tracking code which I adapted to use inside

SatSked.  I've found that longitudes west of longitude 0 (England) should be

entered as a negative number.  Longitudes east of longitude 0 should be

entered as a positive number.  This appears to match the way InstantTrack works.

Message Sending Utility:

-----------------------

To create a message to be uploaded to a specific satellite, press the "M" key

from SatSked.  You will be prompted for the destination ham call, title,

keywords, expiration time, and optionally the message filename.  If

useTimeMsgName is set to 1, the filename will be the current time with the

extension .MSG.

Next you will be placed in the editor specified by the parameter "editor" in

SATSKED.CFG.  The source and destination ham calls will be entered into the

file, along with the time and date.  This message header may be removed with

the "addMessageHeader" parameter in SATSKED.CFG set to zero.  I personally

hope you don't exercise this option as my hard disk will fill up with messages

that I don't know the sender, recipient or time of creation.  If you're worried

about the room it takes, compress the file or delete your signature!

If the destination call exists in the file USERLIST.TXT, (renamed from

UO22USER de JA6FTL) which you should locate in your SatSked directory, the

corresponding ham's name will be inserted in the message file for you.  To the

bottom of your message will be appended the file "suffixFile" as

set in SATSKED.CFG.

When you exit, you will be given the option of sending along additional files

with the text file you just created.  You may enter a path to the desired

drive and directory where the file to upload is located.  Then enter the

filename from the displayed directory, wildcards may be utilized with

the filename.  Note, you will not be able to send extra files if you

have selected no compression as your default "compressor".  Also I've added

the mask feature to select display of files that match a certain

specification.

You must have all the files you want to send in one directory.  So you

can append just one file, or a group of files with a wildcard.  If you

have a group of unrelated files to send, place them in an otherwise

empty directory by shelling to dos first before you send the message.

The directories where you would like the xxxxxx.out files to be sent when

creating a message is specified by the last parameter in SATSKED.PRI, and is

described above.  The directory should contain the complete path, and no

trailing \ character. Note that the message sender will not work if the path

is empty for that satellite.

If it is desired to save the text part of each outgoing messages, the

"msgDir" parameter should be specified in the SATSKED.CFG file.  All

outgoing messages will be copied to this directory.

Message Compression:

-------------------

SatSked's messages can be sent uncompressed (default), zipped (zip)

or lharc'ed (lzh).  Also you can specify "auto" which will try all three, and

send the smallest resultant file.  If the source filesize is less than

"compressionThreshold" bytes long, the uncompressed version will be sent

regardless.  The idea behind this is to send the smallest file to the

satellite.  If the message is a small one, the convenience of not needing to

decompress it outweighs the savings of a few bytes of "ramdisk" space on the

satellite.  The default compression is 1000 bytes

Dumb Terminal:

-------------

By pressing "T", you will be placed in a basic terminal mode, with the comm

port parameters set in SATSKED.CFG.  You can talk to any port, at any address,

any interrupt number, and any baud rate.  These parameters are set by

commPort, cpAddress, cpInterrupt, and baudRate respectively.  Also tab

characters may be expanded by setting expandTabs to the desired length.

If the parameter addUploadLF is set to 1, a LF character will be added to

the carriage return when each line is uploaded.  This is the way that

SatSked versions up to 1.7 operate.  Now this parameter now defaults to 0, and

no linefeed character will be uploaded.  This should fix a problem when you

try to upload keplerian elements to the TrakBox.

You may log any incoming text to disk by pressing F3, upload an ASCII file

with F2 and return to SatSked with F1.  When you upload a file, the parameter

ASCIITransferDelay delays transmission by N milliseconds after each carriage

return is sent.

Pressing F3 asks you for the file name to save the downloaded data in.  If

you hit carriage return, SatSked saves the data in SATSKED.LOG.  If this file

already exists, the new data will be appended to the end of the existing

file.

SatSked has a clock in the lower left hand corner. This clock will

flash if SatSked meets the criteria to sound the beeper.  This indicates that

the current pass would be starting if you leave the dumb terminal with F1.

There eight two programmable function keys, F4 through F12.  These keys may

contain up to 40 characters, and are specified by the desired text in

quotes following the parameter name termF#key, where # is a number from

4 to 12, in SATSKED.CFG.  If you wish the function key to transmit a

carriage return (0x0d), place a ^ character inside the quotes, at the end of

the text to be transmitted.  A couple of examples:

termF4key "C 8J1JBS^"

termF5key "connect "

termF6key "display z^"

Trakbox Support:

---------------

A second dumb terminal can be accessed by pressing control T.  The intention

is for this terminal to be used with the TrakBox if you have one.  The setup

parameters for this terminal begin with tb, and are functionally the same as,

but are separate from the other terminal parameters.  If you don't use a

TrakBox, you can use this terminal to access a different comm port than the

primary terminal if you wish.

If tbTrackOnExit is set to 1, the same comm port as configured above will be

used to start the TrakBox in it's single track mode.  SatSked sends an ascii

"q" to the TrakBox, to return to the main menu, then then selects option 1,

Real Time Tracking.  SatSked watches the response from the start of this menu

for the position of the desired satellite to track.  If that bird is present,

SatSked selects the proper position.  If not, the TrakBox is sent back to the

main menu, and the TrakBox start tracking function is aborted.  SatSked will

present an error message, and continue on with the pass.

If the parameter tbupdateClockOnExit is set to 1, then SatSked will use option

5 to set the TrakBox clock to UTC time, from the dos clock.  This function

may also be performed at any time manually by pressing "h".

If the paramter tbAutoKepUpdate is set to 1, when a new keplerian element

file is found, the new keplerian elements are uploaded to the TrakBox.  The

frequency information file will then be uploaded automatically.  For this to

occur, the file containg frequency information and position, FREQ.DAT,

must be located in the SatSked directory.  This function may also be performed

at any time manually by pressing the "z" key.  Be careful with this option!

If a garbage keplerian element file is chosen automatically, the TrakBox's

data base could be lost, resulting in subsequent lost satellite passes.  Also

newer versions of the TrakBox firmware do not support AMSAT format elements.

If you wish to manually start the TrakBox tracking a satellite, press the

control and "l" keys simultaneously.  The enter the exact satellite name and

hit return.  Note that this satellite must be found in the first 16 positions

of the TrakBox, or the function aborts.

If you wish to stop the trakbox tracking when SatSked starts, or returns from

a pass, set stopTBAtStart to 1 in SATSKED.CFG.  SatSked will send a 'q'

and a carriage return out the trakbox comm port when it starts back up.

SatSked Keys:

------------

There are five menu pages that are accessed by hitting the Tab key to

go forward, and the shift-Tab key to go backwards.  All of the functions

listed in the menu are alive outside of that menu page.  In other words you

don't have to see it to do it.  SatSked now saves the active page before

exiting, and restores it upon program startup.

Now there are 12 user command keys (F1-F12), along with a configurable dos <D>

and view-dir <V> key, for a total of 14 user defined functions.  D is intended

to be used for shelling to dos, and V for View-Dir by ZL2TPO.  You of course

can set these to whatever you want with the "dosCommand" parameter, and the

vCommand parameters.  Enclose the userCommand in quotes if you wish to send

a parameter to the desired program:

example:    userCommand "q.exe textfile.txt"

If you don't use all the userCommand keys, F12 and those unused below that

key will shell to dos.

When a pass is after the time set by "beepTime", the pending pass time

flashes on the screen.  A screen saver will run if no keystrokes are heard

after the time limit you set.  The time to the next pass is still displayed, and

"walks" around the screen.

You may page through the "pass window" to examine future passes with the

usual cursor keys.  Also, any conflicting passes are colored differently, so

you can easily notice them and change the abort time if  you want.

Daily Batch file:

----------------

If desired, SatSked will run a specified batch file once per day at a user

specified time.  The start time should be specified with the "dailyBatchTime"

parameter, and the approximate length of time that this file will take

should be specified with the "dailyBatchRunSeconds" parameter.  The name

of the batch file that is run is fixed, and is called DAILY.BAT.

SatSked Operation with DesqView:

-------------------------------

SatSked may be "tuned" for minimum cpu cycles when used with DesqView.  The

parameter "dvSwaps" may be adjusted to allow DesqView to regain control when

SatSked is waiting for a keystroke or a pass to start.  Values from 1 to

32767 are valid.

Dos Clock Error Correction:

--------------------------

Typically, the hardware clock on your motherboard is more accurate than the

dos clock (at least mine is).  The dos clock may also lose time if software

timer tick interrupts are lost due to buggy TSR's, desqView, Microsoft Windows,

etc.  SatSked can be configured to periodically read the hardware clock, and

set the dos clock to that value.  In addition, SatSked can adjust the

hardware/dos clock by a daily offset number of seconds, to correct for a

known drift.  Note that this offset may be positive, or negative.  The offset

parameter is clockAdjustSeconds, and the parameter to periodically update the

dos clock is residentClockMinutes, which is specified in minutes between

updates.

If it is desired to only update the dos clock when SatSked starts a pass, then

set the parameter updateClockOnExit to 1.  The daily offset will still be

utilized, as long as SatSked starts a pass at least once a day.

The way the offsets are handled are as follows:  To achieve n seconds per day

correction, the day is broken down into n segments of time.  When each time

segment has passed, a second is added (or subtracted if negative offset)

from the dos clock.  If the exact number of segment seconds has occured, as

is not usually the case, the number of seconds which should have been adjusted

is issued, and the "effective" last adjustment time is saved to disk for the

next update.

As an example, lets say clockAdjustSeconds is set to 24.  This means every hour

the dos clock should be offset by +1 seconds.  If 2.5 hours elapses between

adjustments, due to no satellite passes until that time, or if

residentClockMinutes is set to a high value, then the clock is offset by +2.

Then, SatSked calculates the the time of adjustment back by a half hour, and

saves this time to disk.  If a half hour goes by and an adjustment is made,

it will appear as if it's been an hour since the last adjustment, and the

clock is again updated by +1 second.

If more than 3 days elapse between adjustments, such as if you shut off your

computer for a few days, no daily offset will be made to compensate for that

off period.  The "offset timer" will start over from scratch.  Thus you must

accurately set the clock if the CPU's been powered down for a long period of

time.  This is done because the hardware clock will drift differently at a

lower temperature.

If your clock is in really bad shape, you can cause SatSked to update the clock

at regular intervals.  Set the parameter residentClockMinutes to the desired

number of minutes between updates.

I've had pretty good success with this method, easily keeping the dos clock

within a second a day.  It's not automatic, you have to keep an eye on it, but

the price is right!

Timezones and SatSked:

---------------------

SatSked determines the proper timezone from the dos environment variable TZ.

This is also used by InstantTrack.  If TZ is not set, it will default to

Eastern Standard Time, which is UTC - 5 hours.  Here are some examples of

what you might put in your autoexec.bat file:

SET TZ=EST5EDT		Eastern Standard Time (default)

SET TZ=CST6CDT          Central Standard Time

SET TZ=PST8PDT          California Time

SET TZ=UTC0             England, or if you use UTC on your dos clock.

SET TZ=AUS-10           VK3 stations.

You can eliminate the string of characters after the offset number, and SatSked

will not revert to daylight savings time.  The time of year that the daylight

savings time switch is made is fixed, and is either set by DOS or the compiler,

I admit to not knowing which.  I do know that you must set your country code

in your DOS CONFIG.SYS file.  This controls if DOS considers summer to be in the

July timeframe, or January.

The offset from UTC in hours that SatSked is currently using is displayed at

the top left of the SatSked screen, mine says UTC-5 for non daylight savings

time.  All pass data in the window will be adjusted to this timezone.

**********WARNING**********

During the switchover to or from daylight savings time you will run into a

problem if you have the full timezone variable set.  I suggest you either

use UTC time, or leave the trailing three characters blank.  If the three

trailing characters are present, SatSked will then assume you were

there to change your dos clock back or forward by an hour at the time of the

change over.  If this is not the case, you will start passes at the wrong time.

For example, set TZ=EST5 will cause SatSked to maintain an offset from UTC of

5 hours, regardless of the time of year.  When you do change the dos clock to

reflect the new time, you should edit the timezone variable and reboot.  In

this case for summer on the east coast of the USA in the northern hemisphere,

setting the timezone environment variable to EST4 will do the trick.

Silent Operation:

----------------

By hitting the "I" key, all sounds from SatSked can be enabled, or disabled.

This will also disable/enable the AOS beeper, in all modes.  The state of the

sound will be saved when SatSked exits.  No longer do you have to listen to

beeps and boops from SatSked all night, just punch the I key before you go to

bed.

Bells and Whistles:

------------------

You can change the colors of the program, change the parameters which

determine when a pass is started, or blocked, or change the number of times a

single pass will be attempted before giving up.

You may also change the sound of the beeper, use a morse code annunciator

to spell out the satellite to be tracked, or make SatSked call a "beep batch

file" at the pending pass beeptime.  Also you can make the beeper function

operate or not on priority zero satellite passes (these will not generate

track.bat and cause SatSked to exit).  The beeper will only actuate the first

time a particular pass is about to start.  On subsequent passes, the beeper

won't sound.

There's also some other features or options that are contained in the

SATSKED.CFG file that are not explained in this document.  You should take

a moment to read that file.

Some Other Ideas:

----------------

You could edit the PASS.TXT file and create your own fake satellite pass,

setting the name to one which will call a special batch file.  This file could

update your dos clock at a particular time of the day, ring your phone at

work to let you know that your house hasn't burned down from sticking rotors,

or close a relay and shock your dogs!  Make sure that you place the appropriate

maximum runs entry in your SATSKED.PRI file for that event (usually set to 1).

Comments:

--------

SatSked was written using Borland C++ version 3.1, It was compiled for the

generic 8086 processor, and will use a coprocessor if present.

Please report any bugs to me promptly.  Include the offending portion of

PASS.TXT, a description of the error, the time it occurred, and anything

else that you think might help to fix it.  I can be reached on UO-22, KO-23,

KO-25, AO-16, LO-19, wa2n@amsat.org, Compuserve, account number 73055,1733.

Thanks:

------

To all my beta testers, who have helped me develop this program from

1993 through 1995, I wish to extend my sincerest gratitude.  Without you,

SatSked wouldn't be half the program it is now.  Thanks for all the ideas

you've contributed, and the help you gave me sorting out the bugs.

Good luck tracking!

73's  de  WA2N

Wayne D. Roth

Appendix A, using SS.BAT with SatSked to loop through multiple passes:

----------------------------------------------------------------------

To maintain compatibility with the original version of SatSked, setting the

parameter exitToTrack to 1 will cause SatSked to terminate when the AOS time

arrives.  Before termination, the file TRACK.BAT will be generated as described

earlier in this documentation.  A "master" batch file is used to call track.bat,

then restart SatSked again.  I'll call this batch file SS.BAT for this batch

file in the following discussion:

If  you put SS.BAT in a directory that is included in the dos search path you

can start SatSked from anywhere by typing "SS".  The comments column is

not in any of  the files, it's just here for illustration.

******SS.BAT********************Comments***************************************

C:				Changes to the drive with SatSked

CD \SATSKED                     Changes to the SatSked Directory.

SATSKED                         Runs the SatSked main program

IF ERRORLEVEL 1 GOTO CLEANEXIT  If X is pressed from within SatSked, this line

will prevent the recursive call to SS.BAT from

being made.  This allows you to easily exit

from the "loop".

CALL C:\SATSKED\TRACK.BAT       After SatSked Terminates, track.bat is

"called" and executed.

C:\BATCH\SS.BAT                 Since a "call" was made to track.bat, execution

returns to SS.Bat, now we "execute" SS.BAT

again, and the cycle starts over.

:CLEANEXIT                      This is known as a "label" for the goto statement.

*******************************************************************************

Note that this runs over and over until you exit from SatSked with the X key.

When you chain batch files as shown in the call to track.bat, you have the

option of returning, or passing control back the original batch file.  To

return to the first batch file, use CALL filename.bat.  To sever the chain,

just type the name without the call statement.  You can continue to pass

control from file to file if you wish, the process is not limited to 2 levels.

Appendix B -- Example Satellite Element File Formats:

(Copied from N6NKV's InstantTrack documentation)

-----------------------------------------------------

NASA Format

-----------

This is the format used by NASA to distribute satellite elements in their

"NASA Prediction Bulletin".  The origin of the format is unknown.  Some old

NORAD reports refer to this as T-card format.  NASA documents often call it

the "2-line" format.

InstantTrack expects a file which contains groups of 3 lines:  One line

containing the satellite's name, followed by the NASA 2 lines of numbers.

Files of this format are distributed by T.S.Kelso to two nationwide computer

networks, and end up on BBS's and electronic mail systems around the country.

On usenet these can be found in the rec.ham-radio newsgroup.  On internet, in

the INFO-HAMS digest.  If you don't have access to usenet or internet, they

are also available on his telephone BBS "The Celestial RCP/M" (513) 427-0674.

NASA format files look like this...

OSCAR 10

1 14129U          88230.56274695 0.00000042           10000-3 0  3478

2 14129  27.2218 308.9614 6028281 329.3891   6.4794  2.05877164 10960

GPS-0008

1 14189U          88230.24001475 0.00000013                   0  5423

2 14189  63.0801 108.8864 0128028 212.9347 146.3600  2.00555575 37348

Each number is in a specified fixed column.  Spaces are significant.

The last digit on each line is a mod-10 check digit, which is checked by

the program.  The program also checks the sequence numbers (first column),

and checks each orbital element for reasonable range.  This is a very good

set of checks, so this format is very safe, and robust.

The following description, which i obtained from T.S.Kelso, describes the

NASA format in detail...

From: tskelso@ut-emx.UUCP (TS Kelso)

Subject: NASA Prediction Bulletin Format

Date: 21 Aug 88 16:49:21 GMT

Organization: The University of Texas at Austin, Austin, Texas

Data for each satellite consists of three lines in the following format:

AAAAAAAAAAA

1 NNNNNU NNNNNAAA NNNNN.NNNNNNNN +.NNNNNNNN +NNNNN-N +NNNNN-N N NNNNN

2 NNNNN NNN.NNNN NNN.NNNN NNNNNNN NNN.NNNN NNN.NNNN NN.NNNNNNNNNNNNNN

Line 1 is a eleven-character name.

Lines 2 and 3 are the standard Two-Line Orbital Element Set Format identical

to that used by NASA and NORAD.  The format description is:

Line 2

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

10-11     International Designator (Last two digits of launch year)

12-14     International Designator (Launch number of the year)

15-17     International Designator (Piece of launch)

19-20     Epoch Year (Last two digits of year)

21-32     Epoch (Julian Day and fractional portion of the day)

34-43     First Time Derivative of the Mean Motion divided by 2.

or Ballistic Coefficient (Depending of ephemeris type)

45-52     Second Time Derivative of Mean Motion divided by 6. (Blank if N/A)

54-61     BSTAR drag term if GP4 general perturbation theory was used.

Otherwise, radiation pressure coefficient.

63-63     Ephemeris type

65-68     Element number

69-69     Check Sum (Modulo 10)

(Letters, blanks, periods = 0; minus sign = 1; plus sign = 2)

Line 3

Column     Description

01-01     Line Number of Element Data

03-07     Satellite Number

09-16     Inclination [Degrees]

18-25     Right Ascension of the Ascending Node [Degrees]

27-33     Eccentricity (decimal point assumed)

35-42     Argument of Perigee [Degrees]

44-51     Mean Anomaly [Degrees]

53-63     Mean Motion [Revs per day]

64-68     Revolution number at epoch [Revs]

69-69     Check Sum (Modulo 10)

All other columns are blank or fixed.

Note that the International Designator fields are usually blank, as issued in

the NASA Prediction Bulletins.

AMSAT Format

------------

There are several very similar formats generated by several different people

that seem to be called "AMSAT" format.  The following format as distributed in

an ARRL bulletin is supported by SatSked...

KEPLERIAN BULLETIN 77   ARLK077

FROM ARRL HEADQUARTERS

NEWINGTON CT  SEPTEMBER 24, 1988

TO ALL RADIO AMATEURS

Satellite: oscar-10

Catalog number: 14129

Epoch time:      88248.53312992

Element set:     353

Inclination:       27.1605 deg

RA of node:       306.2255 deg

Eccentricity:    0.6029797

Arg of perigee:   333.9978 deg

Mean anomaly:       5.4273 deg

Mean motion:    2.05877131 rev/day

Decay rate:        4.4e-07 rev/day sq

Epoch rev:            3933

Appendix C:

-----------

Files and Placement:

--------------------

Here is how I envision setting up SatSked to be used with PB and PG to track

two of the digital birds in space today:

Directory:   Filename:          Function:

----------   ------------       -----------------------------------------------

C:

����SATSKED                     SatSked directory, it must be in the dos path!!

�                             for these .EXE / com files to be found when

�                             running from a satellite subdirectory.

�        AO-16.BAT          Runs specific commands to track AO-16

�        BEEP.BAT           Set externalBeepPath in SATSKED.CFG to start

�                             the desired beep task before a pass runs.

�        KO-23.BAT          Runs specific commands to track KO-23

�        KEPORDER.TXT       Contains desired order of birds in sat. window.

�        LHARC.EXE          LZW compression utility, used by msg sender.

�        ORBS147N.ELE       Two line keplerian element file.

�        PB.EXE             Comm. pgm to access satellite broadcast mode.

�                             Used to download files, and get directories.

�        PG.EXE             Comm. pgm to access satellite "connected" mode.

�                             Used to upload a files.

�        SATLINK.EXE        Comm. pgm by NZ3F which replaces PB and PG.

�        SATSKED.EXE        SatSked executable.

�        SATSKED.PRI        Satellite priority file for SatSked

�

�        SLxxxxC.CFG,       Config file for SatLink for satellite xxxxx.

�                             A character in posistion C will only be

�                             present if SatSked is started with that

�                             character as it's first parameter.

�             or

�        YYYYYYY.ZZZ        User specified filename for satlink

�                            configuration,  extracted from the file

�                            SATLINK.DIR

�        SSCLEAN.BAT        This file run at end of pass, used for whatever

�        SS.ICO             Windows icon file,  Thanks to W6SHP.

�        STUFFIT.COM        Keyboard TSR used to abort passes early.

�        STRTPBPG.EXE       This executable starts pb/pg / stuffit.com

�                             from the satellite subdirectory.

�        TED3.COM           Default text editor.

�        USERLIST.TXT       JA6FTL's user list, message sender finds

�                             station names from this file if present.

�        PKZIP.EXE          ZIP Compression utility used by message sender.

�

����AO16                    Directory where AO-16's specific files exist.

�                             Execution will transfer here by AO-16.BAT.

�        PG.CFG             config. file that PG.EXE uses on AO-16

�        PB.CFG             config. file that PB.EXE uses on AO-16

�        PB.EQN             Equation file used by PB on AO-16

�        PBHELP.HHH         Help file that PB.EXE accesses.

�        PGDONE.TNC         TNC file used by PG.EXE AO-16 after a pass.

�        PG.TNC             TNC file used by PG.EXE AO-16 before a pass.

�

����KO23                    Directory where KO-23's specific files exist.

Execution will transfer here by KO-23.BAT.

PB.CFG             config. file that PB.EXE uses on KO-23

PB.EQN             Equation file used by PB on AO-16

PBHELP.HHH         Help file that PB.EXE accesses.

PGDONE.TNC         TNC file used by PG.EXE KO-23 after a pass.

PG.TNC             TNC file used by PG.EXE KO-23 before a pass.