MOSH.DOC

---------------------------------------------------------------------

MOSH vers. 3.2, May 1996

In QBX Basic Compiler

by

SM6KFY PETER HULTHE

Address

***  Packet Mail  ***

SM6KFY @ SK6YW.O.SWE.EU

***  Snail Mail ***

Box 5014

426 05 VSTRA FRLUNDA, SWEDEN

---------------------------------------------------------------------

With thanks for the timer routines to:

*** Esko Lehtinen, ELogik Company in Gothenburg, Sweden.

With thanks for revising the English to:

*** G0OIK Peter King in Flamstead, Great Britain.

---------------------------------------------------------------------

This programme was written to control the rig when sending CW to my

friend Peter/G0OIK. With this program your concentration can be

directed towards producing a good chat, while the machine takes over

routine keying.

It was made to be as short and simple as possible, and was intended to

be run from DOS. It is possible to use a very simple PC, but a hard

disk makes life easier.

It is not recommended to be run from any integrated environment like

Windows, Windows 95 or OS2. Only with DOS it gives the very exact

timing of the Morse code that it was constructed for. It is possible

to run a very fast CW with this programme.

With a PC and a tape recorder it is possible to make your own CW

training tape. You can also practice the code with any text written in

standard DOS formate.

When sending or receiving with your rig, you simply write on the

keyboard. You can always (and as well as that) go over to the CW key

whenever you like. What you write on the keyboard is stored in the

log-file, which you can save or not at the end of your session as

you like.

There are many nice CW programmes written. This is an attempt to make

a new one for all those who want a simple but efficiant programme,

filling the gap between the most basic and the more complicated

programs.

---------------------------------------------------------------------

The head programme (MOSH.EXE) will automatically read the text file

(MOSH.CFG) during start-up.

By making changes in MOSH.CFG with a text editor (like the EDIT

supplied by DOS 5.0 and higher) you are able to make your own MOSH.

Points 1 - 12 will show what can be done.

1.  Define your own characters (like national characters, KN etc.).

22 of the more seldom used keys (like {, (, ], [ etc.) can be

programmed to your own "Morse" code.

2.  Set the CW-parameters to your own taste.

The length of Dits, Dahs and delays can be programmed, making

your Morse code personally coloured.

3.  The default speed setting is done by MOSH.DEF.

Speed can then be adjusted up/down with the Left/Right arrows

or be set to "normal" by Home.

With some PC:s older than the 286, the speed will not always be

correctly reported on the screen. A calibration can then be done.

For a calibration of speed, adjust to a reading of 15 wpm (words

per minute). Then run the file PARIS. To do this, hold down the

ALT key and type "F". You will then be presented with a list of

files. Enter the file name "PARIS". The word "PARIS" will be sent

in CW to your loudspeaker 10 times.

The 10 "PARIS" should take exactly 44 seconds. With some older

PC:s, the wpm reading may prove to be a little slower speed than

15 real wpm. If this is the case you may chose a higher wpm figure

and make a note of the difference.

4.  The loudspeaker sound frequency is determined by the MOSH.CFG

file, but it can also be changed when running the programme. The

Up/Down arrows will increase/decrease the frequency in steps by

200 Hz for each touch. If the frequency is too high to the

loudspeaker, you will not be able to hear it, but the rig will

still be controlled.

PgUp and PgDn give 20 kHz resp. 200 Hz.

With End on your keyboard, the frequency is brought back to the

normal (default) value again.

The loudspeaker is stopped/started with the Del key. In the

silent mode the rig is not affected by typing on the keyboard,

but the "silent text" will be stored in the log-file (if used).

This is a way to use the computer as a typewriter, during the

reception of CW or before a session, to make personal notes of

frequency, QRM level etc.

5.  Define short standard texts to be sent using the shift/function

keys from within the MOSH.CFG file.

Up to 12 different messages ("quickies") can be programmed.

You may define the texts as you like. Max. length is 255

characters. Here some proposals:

Shift/F1  : Own call-sign key

Shift/F2  : A commonly used counter-station

Shift/F3  : A short call to the commonly used counter-station

Shift/F7  : CQ-call

Shift/F10 : Over-and-out

There are three "computer signs" that can be used when writing

these short standard texts. Their use will add a time of day

indication in the log.

By holding down the right hand ALT on the keyboard and typing 157

on the numerical keypad you will produce a funny "Y" sign. When

used at the end of the text, this will produce the K (Over-anyone

to answer) code in CW, and also leave a time mark in the log.

Using ALT and 158 gives something like "Pt", producing a KN (Over-

specified station to reply only) again with a time mark in the

log.

ALT/159 gives a "", and is used to produce a time mark only,

and will not give any audible code.

6.  Define the directory name for texts to be sent from, floppy or

hard disk. The directory for these texts should be defined in the

MOSH.CFG file.

Files of up to 10 Kb of text can be sent in any speed you choose.

ALT/F is used to send a text from floppy or hard disk, as was

explained in 3. above.

Texts you want to send can be written using the normal EDIT

function under DOS and be stored under any legal filename.

If you want to stop the text sending, press Esc to quiet it.

7.  Define directory names for the log file and files to send.

You may have a log-file generated after using MOSH. The file will

be placed in a directory you may choose in MOSH.CFG.

8.  The choice of COM-port is the last parameter in MOSH.CFG.

It is good to have a COM-port dedicated only to MOSH, but it is

possible to combine the controlling of MOSH and other hardware

using the same port.

9.  Also some other details are controlled by MOSH.CFG (like the

colours of the written text on the screen, messages etc.).

If you try to run MOSH32.EXE without the MOSH.CFG it is possible,

but some useful functions will not be present. Please note that

the two files must be kept in the same directory.

10. Other options in the programme are some special keys:

The BackSpace key produces a series of Dits (short signs). It

writes "Rub" on the screen and a text "RubOut" in the log-file.

The keyboard buffer can be deleted (made empty) by Ctrl/Pause if

you want to rub out what is yet not sent. The Ctrl/Pause is

sometimes very valuable to use for deleting mistakes. Please try

to remember it and use it!

The Ctrl/K key writes a "K" on the screen and in the log-file,

and makes the "K" Morse code. It also gives a time-mark in the

log-file for the next time a key is pressed.

The Ctrl/O (not the figure!) writes a "Kn" on the screen and in

the log-file, makes the "KN" code plus a time-mark in the log-

file.

The Ctrl/T gives a time-mark in the log-file next time a key is

pressed.

The Ctrl/U writes "Ok!" on the screen and in the log-file and

makes the Understood code (...-.). Also the exclamation mark "!"

can be used for that.

Ctrl/R clears the screen.

Alt/L locks the key ON (for aerial tuning purpose). Another Alt/L

(or any character key) will unlock it.

11. To send small texts in Beacon mode.

The Alt/B key starts/stops the beacon function, where you define

own beacon text and beacon delay. Max. length is 80 characters.

The standard texts can be used as beacons.

One word of warning: This function may be illegally used if you

lack a beacon permission and use it for on air transmission.

12. To stop the programme.

The programme can always be stopped using ALT/X (also Esc works).

Help with the programme is always available with the F1 key (also

Alt/H). The Help Menu can always be consulted during the running of

the programme. There is no need to "escape" from help, just carry

on typing.

---------------------------------------------------------------------

Interfacing hardware.

There are two ways of interfacing the computer to the rig.

A. The loudspeaker method.

To make a simple tap of the PC:s loudspeaker and to have the audio

converted into a conducting path will probably work well. An

optocoupler (OC) is excellent for a job like that.

An OC is a light emitting diode (LED) and a photo transistor, housed

in the same case, looking like an IC, often having 6 legs.

The LED part (legs  1 - 2) is connected to the computer's loudspeaker

via a 150 Ohm resistor. A 33 nF capacitor is connected across the LED

terminals of the OC. The TL 111 and 4N25 OC:s were tried.

The transistor part of the OC (legs 4-5 are used, the emitter and

collector terminals) is connected to the rigs' key connections in such

a way that the key current will flow through the transistor of the OC

when the LED is lit by the loudspeaker current. A 1 nF capacitor

across the key terminals is used to reduce any leaking RF energy.

This arrangement has worked well with an old XT and a 486 in my

shack, together with a Yaesu FT101ZD and an ICOM IC735.

However, with some PC:s and some rigs there can be problems.

The generating of sound is not standardized within the PC concept. If

the sound signal is too low, or if a polarizing voltage appears at the

connectors to the loudspeaker, an extra transistor before the OC may

be needed. I have recently tried a Victor 286 which demanded for that

solution.

It maybe found with some older rigs that a transistor may be required

between the rig and the OC. Connect the OC leg 4 to the base of the

transistor and the OC leg 5 to the collector (Darlington connection).

The key connectors are then wired to the emitter and collector of the

transistor (only one polarity works!).

B. The COM port method.

An easy way to interface the PC to the rig is to use one of the

COM-ports (sometimes called RS-232, or serial ports). They are male

25 pin or 9 pin connectors at the rear of the PC, used for mice and

modems. As it is good advice to electrically separate the PC from the

rig I have been using an opto coupler to let the PC control the key

line.

TxD found in the COM-port is a connection used for data transfer.

It is pin 2 in the 25-p or pin 3 in the 9-p D-sub. Ground (GND) is

pin 7 in the 25-p or pin 5 in the 9-p D-sub. TxD is at about -12 V

negative with no tone present. When a CW tone is produced by MOSH, the

TxD will go to a positive potential of +12 V on most PC:s. With some

PC:s (particularly lap-tops) the potentials are lower (mostly + and

- 5 V), but this should not change the function.

Connect TxD to the OC LED anode (pin 1) via a diode and a 2.2 kohm

resistor. Connect the OC LED cathode (pin 2) to GND of the COM port.

There will be a flow of about 5 mA from TxD to GND through the LED,

when a CW tone comes. This will make the OC transistor conduct.

For many rigs this conductance is enough to produce a sufficient key

current to switch the rig into transmit mode.

If not, a second transistor can be tried, making a Darlington stage

together with the OC transistor. If also this fails, the voltage

between the key terminals is not sufficient to supply the transistor.

Then a 3 V external voltage in series with a 1 kohm resistor can

help.

A capacitor of 0.1 uF is likely to be placed over the LED legs of the

OC, and one of 1 nF over the transistor's legs to the rig's key line.

The above methods have all been tried, and have solved all related

problems I have been faced to up till now.

It has been observed that the the interface used for HamComm (by

DL5YEC) can use the same RS-232 port as MOSH. It will then be

possible to use any of the two programmes to control the CW keying

function, with the same opto-coupler CW interface.

---------------------------------------------------------------------

SUMMARY OF CONTROL FUNCTIONS:

Arrows Up/Down       -Raise/Lower audio freq' from the PCs loudspeaker

PgUp                 -Highest setting, 20 KHz, gives quiet sending)

PgDn                 -Lowest  setting, 200 Hz)

End                  -Default setting of audio freq'

Arrows Left/Right    -Increase/Decrease speed of CW

Home                 -Default setting of CW speed

Delete               -Toggles between TX/RX. (In TX what is typed is

sent, in RX what is typed is not sent, but is

recorded in the Log-file).

Backspace            -Sends error character and writes "Rub" on

screen.

Ctrl/K                Sends "K" (-.-) & makes time mark in log.

Ctrl/O               -Sends "KN" (-.--.) & makes time mark in log.

Ctrl/T               -Makes a time mark in the log.

Ctrl/R               -Clears the screen.

Ctrl/U  !            -Sends "Understood" (...-.) & writes "OK" in

the log.

Ctrl/Pause           -Clears the buffer and so stops sending.

Shift/F1             -Sends pre-defined texts (like a CQ-call or what

---                   you may have chosen.

Shift/F10

Alt/B                -Starts/Stops beacon mode.

Alt/F                -To send pre-written text files.

Alt/H                -Shows help screen.

Alt/X                -Exits the program.

F1                   -Shows the help screen.

Esc                  -Stops the sending of text files

-Also exits the program when no text file is

being sent.

---------------------------------------------------------------------

This programme is FREEWARE. It allows you to use it as much as you

like, as long as you want.

If you find any errors, please tell me about them!

And if you have got any bright idea about changes, don't

forget to drop a few lines to me about that!

73 de SM6KFY / Peter