BPQ Yapp File Server Program For IBM-PC/Clones

==============================================

Copyright Jon Welch - Ver 1.00 04/07/91

---------------------------------------

Introduction

============

The BPQ Yapp File Server program is a software package which

provides a file server on a BPQ Node to enable users to upload or

download files from it using the YAPP file transfer protocol.  26

different  file  areas can be defined with access  restricted  to

upload  only,  download  only  or full  access.  Help  files  are

available  on  all commands plus an extra info  file  for  system

details.   The  system  name  and  command  prompts   are   fully

configurable by the user.

DISCLAIMER OF WARRANTY

THIS  SOFTWARE  AND  MANUAL  ARE PROVIDED  "AS  IS"  AND  WITHOUT

WARRANTIES  AS  TO PERFORMANCE OF MERCHANTABILITY  OR  ANY  OTHER

WARRANTIES WHETHER EXPRESSED OR IMPLIED.  BECAUSE OF THE  VARIOUS

HARDWARE AND SOFTWARE ENVIRONMENTS INTO WHICH THIS PROGRAM MAY BE

PUT, NO WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE IS OFFERED.

BPQYAPP V 1.00                  1                        04/07/91

C O N T E N T S

===============

Chapter                                           Page

-------                                           ----

1 System Requirements                            3

2 Installing The Software                        4

2.1 System Files and Usage                     4

3 Configuring The Software                       5

4 Setting up the Yapp File Server                7

5 Using the Yapp File Server                     8

Appendices

----------

A - My BPQCFG.TXT File                          10

B - Recognised Users                            16

Distribution Restrictions

=========================

The  BPQ  Yapp  File Server program  is  NOT  public  domain

software  but  I am making no charge for you to use  it.  If  you

wish, you can make a contribution to the development of this  and

future  amateur radio software which I am currently  writing  but

feel under no obligation to do so.

If you want to contribute or just contact me with  comments,

asking for help and advice etc. please write to :

Jon Welch,

50 Quarrydale Road,

Sutton In Ashfield,

Notts. NG17 4DR.

Great Britain

If  you  are  writing from  outside  Great  Britain,  please

remember to include your country in your address so I know  where

to reply to if necessary !

BPQYAPP V 1.00                  2                        04/07/91

1 System Requirements

=====================

The software is designed to run in conjunction with the  BPQ

Node  software  and  Deskview and as such, will  not  work  as  a

standalone YAPP File Server package.

The software will work on any 80x86 computer running  MS-DOS

3.00  or  above  with  a  window  size  of  at  least  36K  under

Deskview.

The software has been tested on BPQ V 4.01 and Deskview 2.31

but should work with any compatible version.

BPQYAPP V 1.00                  3                        04/07/91

2 Installing The Software

=========================

To install the software you will need to copy all the  files

from  the  distribution file to a directory on a floppy  disc  or

your hard disc.

You will also need to modify your current BPQCFG.TXT file to

add the yapp file server as an application into the node. This is

done  my  modify  the  applications line in  the  file  and  also

creating a port definition for the program. See the example of my

current BPQCFG.TXT file in Appendix E for help on doing this.

In   more  detail,  you  need  to  add  a  keyword  to   the

applications  line which the user will enter to access  the  Yapp

File Server. There is room for eight on the applications line  so

find a spare slot and add it in. Each of the keywords is numbered

starting  at $01, then going up to $02, $04, $08, $10,  $20,  $40

and $80 for the other seven. In my example below, my YAPP keyword

is  the fifth in the line and so it's number or APPLMASK is  $10.

This  needs  to  be defined in the  port  configuration  for  the

program as shown below also.

TNCPORT                       APPLICATIONS=PMS,,*SYS,MH,YAPP

COM=3

TYPE=TNC2

APPLMASK=$10

APPLFLAGS=$00

ENDPORT

Please  refer  to the relevant documention that  comes  with

your copy of the BPQ software for more information if required.

You  will also need to create a DVP file for Deskview and  a

sample one is enclosed for you to work from.

2.1 System Files and Usage

--------------------------

The files comprising the system, where they are located  and

their function are as follows :

BPQYAPP.CFG - Configuration file for program (mandatory)

BPQYAPP.HLP - Text file sent for H command   (optional)

BPQYAPP.INF - Text file sent for I command   (optional)

BPQYAPP.EXE - Main Program

YS-PIF.DVP  - Sample configuration file for Deskview

BPQCFG.TXT  - My BPQ configuration file

BPQYAPP V 1.00                  4                        04/07/91

3 Configuring The Software

==========================

The configuration for the program is stored in a single file

called BPQYAPP.CFG. This file is a standard text file and  should

be  edited  with  a  text editor that  does  not  add  formatting

characters to the text (such as Wordstar in non-document mode).

The format of the file is given below :

Line 1

------

This  tells the computer which virtual COM port your TNC  is

connected to in the BPQ Node software. It can either be from 1 to

16  depending  how you BPQCFG.TXT is configured.

Line 2

------

This tells the computer which application number the program

is  based on it's position in the BPQCFG.TXT  applications  line.

For each of the available positions 1 to 8, this number will be :

1, 2, 4, 8, 16, 32, 64 or 128

Line 3

------

This  line  is the name you wish to give to your  Yapp  File

Server.  This  is  displayed on the welcome screen  when  a  user

connects to your file server.

eg.      Jon's Megga Fabbo Yapp File Server

The  following lines identify the file upload  and  download

areas  you  wish to set up. Each line comprises of  five  fields,

each separated by a comma.

Field 1 - File Area Designator. This is a single letter from

A  to Z which uniquely identifies the area. You need not  specify

the areas in alphabetical order.

Field 2 - File Area Directory. This is the complete pathname

(including  drive if necessary) of the directory  containing  the

files for this area.

Field 3 - File Area Description. This text string is used on

the  W command to tell the user what type of files are  contained

in the file area.

Field  4  - Download Flag. This can be either a Y or  N  and

tells the system if file downloads are allowed from this area.

BPQYAPP V 1.00                  5                        04/07/91

Field 5 - Upload Flag. This can be either a Y or N and tells

the system if file uploads are allowed to this area.

eg.  A,C:\TNC\PMS\DOWNLOAD,Download Area,Y,N

You must terminate the list of file areas with a single line

containing a '*'.

The final line in the configuration file is the prompt  sent

to the user.

eg.        Cmd:B,D,H,I,U,V,W >

A sample BPQYAPP.CFG file is thus :

3

16

Jon's BPQ Yapp Server

A,C:\TNC\PMS\DOWNLOAD\GENERAL,General Download Area,Y,N

C,E:\GRPHCGA,CD-ROM CGA Graphic Programs     89 files,Y,N

E,E:\GRPHEGA,CD-ROM EGA/VGA Graphic Programs 94 files,Y,N

G,E:\GRPHUTIL,CD-ROM General Graphic Programs 47 files,Y,N

H,E:\GAMEADVE,CD-ROM Adventure Games         143 files,Y,N

I,E:\GAMEARCA,CD-ROM Arcade Games            270 files,Y,N

J,E:\GIFS,CD-ROM GIF Pictures            336 files,Y,N

T,C:\TNC\PMS\DOWNLOAD\TNC,TNC Driver Support Files,Y,N

U,C:\TNC\PMS\UPLOAD,Upload Area,N,Y

V,E:\VIRUSERS,CD-ROM Virus Protection Programs,Y,N

*

Cmd:B,D,H,I,U,V,W >

BPQYAPP V 1.00                  6                        04/07/91

3.1 Setting up the Yapp File Server

-----------------------------------

There  is  very  little to do in setting up  the  Yapp  File

Server.

If  you have not done so already, configure the  BPQYAPP.CFG

file accordingly.

You   may  also  want  to  customise  the  main  help   file

BPQYAPP.HLP and the info file BPQYAPP.INF

For  all  your file areas, you can also create  an  optional

'area information' file. Again, this is a plain ASCII text  file,

must  be  called AREA.INF and stored in any file  area  that  you

require. When the user issues a 'W <area>' command, the  AREA.INF

file is also displayed below the header line and before the  list

of  file  names. This feature can be used to tell the  user,  for

example,  that all files in the area are .ZIP files and  must  be

decompressed with PKUNZIP before use.

BPQYAPP V 1.00                  7                        04/07/91

3.2 Using The Yapp File Server

------------------------------

Assuming  you have installed the software correctly,  loaded

the  BPQ code, Deskview, a suitable terminal driver  program  and

opened  up the Yapp File Server program, the Yapp File Server  is

accessed  by  connecting to the node and typing the  keyword  you

defined in the BPQCFG.TXT applications line.

You  will then be welcomed to the file server and  be  given

the command prompt.

All  the time a remote user is logged onto the file  server,

the  sysop still has control over the keyboard in the  window  as

well. This means that the sysop can enter commands as though they

were  coming from the remote user. This can be used to guide  new

users through the program by showing them what commands do  what,

or  the  remote user can be logged off if he  mis-uses  the  file

server etc.

To  close down the file server, simple use the close  option

of Deskview or press Alt X whilst in the file server window.

The  different commands available whilst in the file  server

are :

B - Log off the Yapp File Server.

Disconnecting  has  the same effect. The program  will  then

revert back to waiting for the next user to connect.

D <area> <file> - Download file.

This  is  used to download a file using YAPP  transfer.  The

area  can  be A to Z if setup in the configuration  file  by  the

sysop.

eg. D B HELP.COM

H - Display help file.

This  command will display the BPQYAPP.HLP file to the  user

if one exists.

I - Display info file.

This  command  will  display the  BPQYAPP.INF  file  if  one

exists.   Use   it  to  show  your   stations   hardware/software

configuration and anything else you may wish to tell the user.

BPQYAPP V 1.00                  8                        04/07/91

U <area> <file> - Upload file.

This is used to upload a file using YAPP transfer. The  area

can be A to Z if setup in the configuration file by the sysop.

eg. U A MYPROG.EXE

V - Show version.

This  will  show  what version of the Yapp  File  Server  is

running.

W - List file areas.

This will list the file areas as configured by the sysop  in

the  BPQYAPP.CFG  file. Each area will indicate whether  you  can

download and upload to it.

W <area> (filespec) - List files (matching filespec).

This  will list all the files matching the  optional  filespec

(defaulting to all files) contained in that particular area.  The

area can be A to Z if setup by the sysop.

eg. W A *.ZIP

BPQYAPP V 1.00                  9                        04/07/91

Appendix A - My BPQCFG.TXT File

==========   ------------------

; For Version 3.59

;

;

; CONFIGURATION FILE FOR G8BPQ SWITCH SOFTWARE

;

; The program which converts this text file to binary

; form is currently very crude - be VERY careful when

; editing this file!

;

;

; The order of parameters in not important, but they

; all must be specified - there are no defaults

;

;

;

; Memory Usage. If EMS=1, the system will attempt to put its

; data area into EMS Memory. At the moment it makes no attempt

; to manage sharing of the EMS window, so you must not run any

; other software using EMS.

;

; It seems to work ok when used with Desqview, at least on my

; AST, but it is primarily intended for use with AA4RE BBS, which

; needs a great deal of space.

;

;

EMS=0                           ; dont use EMS RAM

;

; I seem to have problems with systems not using DESQVIEW -

; especially NNA BBS's. The DV calls SHOULD be ignored if DV is

; not loaded, but just in case, if you set DESQVIEW=0, the

; 'Release Timeslice' call wont be used. MAKE SURE that you set

; DESQVIEW=1 if using DV, or performance will be seriously

; affected.

;

DESQVIEW=1

;

;

; Station Identification.

;

; If a user connects to the NODE Callsign or Alias, he is linked

; to the switch code, and can use normal NetRom/TheNet commands

;

; If he connects to the BBS Callsign or Alias he will be connected

; directly to a BBS port. If none are available, the connect will

; be rejected.

;

; Note that of you are running only a NODE (ie BBS=0), or

; or just a BBS (NODE=0), then you must still put in two pairs of

; calls, but they MUST be the SAME.

;

; If you are running both, the calls MUST be different

BPQYAPP V 1.00                  10                        04/07/91

NODECALL=G7JJF-8                ; NODE CALLSIGN

NODEALIAS=IP0423

BBSCALL=G7JJF                   ; BBS CALLSIGN

BBSALIAS=JJFPMS                 ; BBS ALIAS

;

; 'ID' MESSAGE - SENT EVERY IDINTERVAL MINS

;

; WILL BE ADDRESSED FROM THE PORT CALLSIGN (IF DEFINED)

;      ELSE FROM THE NODE CALL

;

; The following format allows KA nodes to include your system in

; their 'Nodes' lists.

;

IDMSG:

Network node (IP0423) running TNCBPQ V1.40 and BPQ Yapp Server V1.00

***

;

UNPROTO=MAIL                         ; DEFAULT UNPROTO ADDR

; 'I' COMMAND TEXT

;

; Note that the bit about needing a port param in connects is only

; necessary if you are running more that one (radio) port.

;

INFOMSG:

G7JJF Packet Switch, Sutton In Ashfield, Notts. IO93IC

Commands are basically the same as NET/ROM, but to connect to

another normal station (not another node), you must specify a

port number before the callsign. Use PORTS command to list

available ports. The PMS command connects you to the associated

Mailbox and the YAPP command takes you to a Yapp File Server

for downloading/uploading of files.

***

; CTEXT - If specified, will be sent when someone connects to

; the NODE ALIAS at level 2. It is not sent on connects to the

; NODE callsign, as this could confuse BBS forwarding connect

; scripts.

;

CTEXT:

Welcome to G7JJF's Packet Switch in Sutton In Ashfield

Type ? for list of available commands.

***

;       Network System Parameters

;

OBSINIT=5               ; INITIAL OBSOLESCENCE VALUE

OBSMIN=4                ; MINIMUM TO BROADCAST

NODESINTERVAL=60        ; 'NODES' INTERVAL IN MINS

IDINTERVAL=15           ; 'ID' BROADCAST INTERVAL (UK Regs require

; an AX25 ID every 15 mins)

;

BPQYAPP V 1.00                  11                        04/07/91

BTINTERVAL=15           ; NO BEACONS

L3TIMETOLIVE=25         ; MAX L3 HOPS

L4RETRIES=3             ; LEVEL 4 RETRY COUNT

;

; I have seen suggestions that L4TIMEOUT should be increased to very

; high values (even as long as seven mins). Although 60 may be a bit

; short in some cases, I STRONGLY advise that you dont go much above

; 120 secs unless you understand ALL the implications.

;

; I've set TTL and TIMEOUT to the values agreed by the latest SYSOPS

; meeting - this does not imply that I agree with them!

;

L4TIMEOUT=120           ; LEVEL 4 TIMEOUT

L4DELAY=10              ; LEVEL 4 DELAYED ACK TIMER

L4WINDOW=4              ; DEFAULT LEVEL 4 WINDOW

;

MAXLINKS=30             ; MAX LEVEL 2 LINKS (UP,DOWN AND INTERNODE)

MAXNODES=120            ; MAX NODES IN SYSTEM

MAXROUTES=35            ; MAX ADJACENT NODES

MAXCIRCUITS=64          ; NUMBER OF L4 CIRCUITS

minqual=10              ; MINIMUM QUALITY TO ADD TO NODES TABLE

BBSQUAL=250             ; BBS Quality relative to NODE - used to

; limit 'spread' of BBS through the

; network to your required service area.

; I've been asked to set a low default to

; encourage you to think about a suitable

; value. Max is 255

BUFFERS=255             ; PACKET BUFFERS - 255 MEANS ALLOCATE AS MANY

; AS POSSIBLE - NORMALLY ABOUT 130,

; DEPENDING  ON OTHER TABLE SIZES

;

;       TNC DEFAULT PARAMS

;

PACLEN=128                      ; MAX PACKET SIZE

;

; PACLEN is a problem! The ideal size depends on the link(s) over

; which a packet will be sent. For a session involving another node,

; we have no idea what is at the far end. Ideally each node should have

; the capability to combine and then refragment messages to suit each

; link segment - maybe when there are more of my nodes about than 'real'

; ones, i'll do it. When the node is accessed directly, things are a

; bit easier, as we know at least something about the link. So, from

; Version 3.11 onwards, there are two PACLEN params, one here and

; one in the PORTS section. This one is used to set the initial value

; for sessions via other nodes, and for sessions initiated from here.

; The other is used for incoming direct (Level 2)       sessions.

; In all cases the TNC PACLEN command can be used to override

; the defaults.

;

; The main reason for changing this now was a request from someone

; wanting to run an HF port.

;

;

BPQYAPP V 1.00                  12                        04/07/91

;

TRANSDELAY=1            ; TRANSPARENT MODE SEND DELAY - 1 SEC

;

;       Level 2 Parameters

;

; T1, T2 and N2 are now int the PORTS section

;

T3=180                  ; LINK VALIDATION TIMER (3 MINS)

IDLETIME=900            ; IDLE LINK SHUTDOWN TIMER (15 MINS)

;

;

;       Configuration Options

;

BBS=1                   ; INCLUDE BBS SUPPORT

NODE=1                  ; INCLUDE SWITCH SUPPORT

;

HIDENODES=0             ; IF SET TO 1, NODES STARTING WITH # WILL

; ONLY BE DISPLAYED BY A NODES * COMMAND

;

;

;

;       'COMBIOS' Port definitions.

;

;       This section defines the virtual TNC's that the code supports.

;

;       I'm afraid this is now rather more complicated than it was!

;

;       You can now define any COMBIOS port to run in normal (TNC2)

;       mode, PK232 HOSTMODE (with varients for UFQ and AA4RE), or KISS

;       mode. KISS mode allows you to run things like TCP/IP, STATS

;       packages, PACSAT Telemetry decode, etc above the switch.

;

;       The basic format is:

;

;           TNCPORT

;               COM=n           ; COMBIOS port number

;               TYPE=TNC2|KISS|PK232/AA4RE|PK232/UFQ

;               KISSMASK=nn     ; Required only for KISS

;               APPLMASK=nn     ; Defaults to 1 (BBS)

;           ENDPORT

;

;       TYPE defaults to TNC2. See file KISSMODE.DOC for full details

;       of facilities available in KISS mode.

;

;       The two PK232 modes are equvalent to the old HOSTMODE=1 or 2,

;       but can now be used on any port. See HOSTMODE.DOC for more

;       details

;       Dont try to define more than 16 ports.

;

;       For my software, you must decide on the port for the PMS to be

;       accessed from. This can be anything from 1 to 15.

;       You must then set this port, plus the following to the settings

;       shown below :

BPQYAPP V 1.00                  13                        04/07/91

TNCPORT

COM=1

TYPE=TNC2

APPLMASK=$03

APPLFLAGS=$07

ENDPORT

TNCPORT

COM=2

TYPE=TNC2

APPLMASK=$08

APPLFLAGS=$00

ENDPORT

;       This is the setting for my BPQ Yapp File Server if you

;       want to install it eventually.

TNCPORT

COM=3

TYPE=TNC2

APPLMASK=$10

APPLFLAGS=$00

ENDPORT

PORT

;       ID=144.625 Mhz 1200 baud

ID=Not Active

TYPE=ASYNC

PROTOCOL=KISS

IOADDR=2F8H                     ; COM1 - FOR COM2 PUT 2F8H

INTLEVEL=3                      ; COM1 - FOR COM2 PUT 3

SPEED=9600

CHANNEL=A

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

BPQYAPP V 1.00                  14                        04/07/91

;

;       2ND PORT OF KPC4/KAM (FIRST PORT IS DEFINED AS ABOVE)

;

PORT

;       ID=432.675 Mhz 1200 baud

ID=144.625 Mhz 1200 baud

TYPE=ASYNC

PROTOCOL=KISS

IOADDR=2F8H

INTLEVEL=3

SPEED=9600

CHANNEL=B       ; this is the special bit for KAM/KPC4

QUALITY=10

MAXFRAME=2

TXDELAY=500

SLOTTIME=100

PERSIST=64

FULLDUP=0

FRACK=7000

RESPTIME=2000

RETRIES=10

PACLEN=120

ENDPORT

ROUTES:

;

;       ROUTES TO LOCK IN

;

;       specify Callsign, Quality and Port. You can now also specify

;       MAXFRAME, FRACK, PACLEN (in that order) to override the port

;       defaults.

;

;

***

;

;       APPLICATIONS SUPPORTED

;

;       UP TO 8 ENTRIES OF UP TO 12 BYTES EACH

;

;       THE FIRST 3 ARE RESERVED FOR BBS, HOST, AND REMOTE SYSOP

;       YOU CAN DEFINE THE OTHERS AS YOU LIKE.

;

;       FOR COMPATIBILITY WITH EARLIER VERSIONS, JUST ENTER

;       APPLICATIONS=BBS

;

;       NAMES STARTING WITH * ARE NOT INCLUDED IN 'VALID COMMANDS'

;       DISPALY

;

APPLICATIONS=PMS,,*SYS,MH,YAPP

;

;

BPQYAPP V 1.00                  15                        04/07/91

Appendix B - Recognised Users

==========   ----------------

Jon      G7JJF

If  you have any problems using the software, first  of  all

please  read this manual thoroughly. The answer to most  of  your

questions  will  be hidden somewhere. If all else  fails,  please

contact me at the address given below or G7JJF @ GB7FLG for  help

and advice.

I  would  also appreciate any input you would care  to  give

concerning  the program.  If you have any ideas or comments  that

would make it a better program, then please let me know.

*****************************************************

**** Remember To Support Your Local Packet Group ****

*****************************************************

Jon Welch,                 G7JJF @ GB7FLG.#23.GBR.EU

50 Quarrydale Road,        CompuServe ID : 100012,2607

Sutton In Ashfield,

Notts. NG17 4DR.

Great Britain

4th July, 1991

BPQYAPP V 1.00                  16                        04/07/91