--- Data Engine Help ---

AUTOLF     LF sent to terminal after CR

AX25LVL    {1 | 2}

AXDELAY    Time added to TXDELAY for repeaters  (10 msec)

AXHANG     Hang time of repeater to inhibit AXDELAY  (10 msec)

BEAC  ON   [{EVERY | AFTER}] time    (1 min)

BPATH      Beacons use this path

BREAK      Enables a return to command mode with break sequence

BTEXT      Text sent to BPATH in UI packet

CONNECT    callsign [via calls]     can be used to reconnect with different path

CANLINE    Char used to cancel a partial line

CANPAC     Toggle char used to send output from TNC to bit bucket

CHECK      No activity time before protocol checks on link  (10 sec)

CMDTIME    Timing spec to get out of transparent mode (sec)

CMSG       {ON | OFF | DISC | PBBS}

COMMAND    Char used to return to command mode

CONMODE    [{CONVERSATIONAL | TRANSPARENT | NONE}] [{DEFERRED | IMMEDIATE}]

CONOK      Enables connections to MYCALL

CONVERS    Enter conversational mode

CPACTIME   Enables PACTIME use in conversational mode

CR         SENDPAC char appended to packets formed by SENDPAC

CSTAMP     Time stamp connects and disconnects

CTEXT      Text sent first to someone else connecting to MYCALL

CWID       [{EVERY | AFTER}] [time] [{KEY | TONE}]   (1 min)

DISCONNECT Disconnect current stream;  D MYPBBS forces PBBS disconnect

DISCMODE   {COMMAND | NONE}

DAYTIME    yymmddhhmm[ss]

DAYSTRING  { mm/dd/yy hh:mm:ss | d.m.y h:mm:ss | mmm d 19yy   h:ss CST | etc.}

DBLDISC    Forces a true disconnect sequence after DISC while awaiting connect

DELETE     Char used to delete last char in line

DIGIPEAT   Allows digipeating through MYCALL

DISPLAY    {ASYNC | CHAR | ID | LINK | MONITOR | TIMING}

DWAIT      Time to allow digipeaters first access to channel (10 msec)

ECHO       Characters from terminal are echoed back

FLOW       TNC will not send data to terminal until input line complete

FLOWR      TNC will respond to START and STOP in command or convers mode

FLOWX      TNC will send XON and XOFF in command or convers mode

FLOWTR     TNC will respond to START and STOP in transparent mode

FLOWTX     TNC will send XON and XOFF in transparent mode

FRACK      Time to wait for ack to packets (FRACK*(2*d+1) sec)

FREERAM    Bytes available in heap

HELP       Read your manual

HID        Auto ID every 9.5 min if TNC is digipeating

ID         Forces an ID packet

INTERFACE  {TERMINAL | BBS | HOST | KISS}

IPATH      Ids use this path

ITEXT      Text sent to IPATH in UI packet

K          same as CONVERS

KISSADDR   Sets the KISS addresses for the two ports

LEDS       Enables front panel LEDS

LFADD      LF char added to each CR char sent in packet

LIDLIST    {NONE | calls | {+|-}call}

MONITOR    master switch

MAXFRAME   Maximum number of unacknowledged I frames outstanding

MAXUSERS   Maximum number of connections allowed

MHEARD     {n | [{LONG | SHORT}]}

MODE       [baud rate][,[parity][,[data bits][,stop bits]]]

MODEM      [auxillary][,[{FULL|HALF}][,baudrate]]

MONLIST    {NONE | ALL | moncalls | ALL [EXCEPT] moncalls | {+|-}moncall}

moncall={call[({FROM | TO})]}

MONMODE    {NONE | ALL | modes | {+|-}mode}

modes={CONNECTED,FILTER,HEADER,PATH,TIME}

MONTYPE    {NONE | ALL | types | {+|-}type}

types={COMMANDS,CONNECTS,DATA,PIDS,RESPONSES,UNPROTOS}

MYCALL     This station's callsign  {call[-n]}

MYALIAS    {call | NONE}  allows digipeating with alias callsign

MYGATE     {call | NONE}  allows cross band digipeating with gateway callsign

MYPBBS     {call | NONE}  allows access to personal mail box

MYREMOTE   {call | NONE}  allows access to remote control

PACLEN     Maximum size of data in I frame  (1-256)

PACTIME    [{EVERY | AFTER}] time    (sec)  when to form packets in trans mode

PASS       Char used to pass special characters as data

PASSALL    Allows packets with incorrect CRCs to be processed

PBBS       Amount of memory (1K increments) set aside for PBBS messages

PBTIMER    Inactivity timer for PBBS

PBPERSONAL Restricts BBS messages addressed to MYCALL and MYPBBS

PERM       {ALL | parameter}  preserves parameters for power up

PERSIST    probability of transmitting during slot time (0-255)

PORT       {0 | 1 | 2}  specifies operation as single or dual port

PTEXT      Initial text sent when someone first connects to PBBS

REDISP     Char used to redisplay partial line

RELINK     Version 2 will try to reconnect after retrying out

REMTIMER   Inactivity timer for remote channel

RESPTIME   Time to wait before acking data packets

RESET      Soft reset;  does not use permed parameters

RESTORE    {ALL | parameter}  restores parameters preserved by PERM

RETRY      Number of retries before aborting

RING       3 bells sent to terminal upon incoming connect

RTEXT      Text string used for remote authorization

STATUS     [LONG]  shows status of active streams [or all streams]

SENDPAC    Char used to form packets in convers mode

SLOTTIME   Time between successive tries of persistance algorithm (10 msec)

START      Char to restart output from TNC to terminal

STOP       Char to stop output from TNC to terminal

STREAMSW   Char used to change streams or ports

STREAMCA   Show callsigns with stream switch

STREAMEV   Show stream switch with every packet

TRANS      Enter transparent mode

TRACE      Dump out received packets in hex

TRIES      Number of retries on current packet

TXDELAY    Time delay between PTT and radio data out (10 msec)

UNPROTO    {call [via calls]} path for UI data frames

UPLOAD     program/data into RAM

USERS      specifies streams to be used by incoming connects

XOFF       Char to stop input from terminal to TNC

XON        Char to restart input from terminal to TNC

*** EOF